Project Files Guide

This project combines plantar pressure, EMG and right-leg exoskeleton perturbation recordings to assess EMG responses, center of pressure (COP) and bilateral load transfer under eyes-open (EO) and eyes-closed (EC) conditions.

1 Folder overview

| Folder | Contents and purpose |
| --- | --- |
| data/ | Experimental CSVs. OE1–OE10 and CE1–CE10 are two numbered trial groups. zero/, left/, right/ and MVC/ contain zero-offset, left/right single-stance load and maximum voluntary contraction calibration. balance/ contains other recordings. |
| figures/ | Folder for manuscript figures, for presenting results and paper layout. |
| linux code/ | network_bridge_node.py: Linux exoskeleton control and UDP bridge for perturbation triggering, joint-angle feedback and Windows communication. Requires ROS 2 and the exoskeleton low-level driver. |
| windows code/ | main.py: acquisition, playback and analysis entry point; backend_*.py: device communication; frontend_gui.py: interface; data_analysis_ui.py: offline analysis. Keep the JSON / STL configuration, calibration and model assets. |

Trial files: Exoskeleton_Data.csv (exoskeleton), Insole_Left.csv / Insole_Right.csv (pressure), emg_dev12.csv / emg_dev15.csv (EMG), and *_imu.csv. Timestamp field: Unix_Time_s.

2 Basic use

Windows environment: Python 3, numpy, pandas, scipy, PyQt5 and pyqtgraph.

Offline analysis: Run python main.py from windows code/. In Offline Analysis Station, manually load zero/, left/, right/ and MVC/ from ../data/, then select a trial. The Linux script is not needed.

Acquisition: Update LINUX_ROS_IP on Windows and WINDOWS_IP on Linux (UDP: Linux 50051; Windows 50052). Check the emergency stop, zero position and limits. Load ROS 2 and start the low-level driver; run python3 network_bridge_node.py from linux code/, then start the Windows application. Record before triggering.

3 Checks before use

Hardware and paths: The Linux script enables the motor on startup; do not run it for offline viewing. New recordings still default to windows code/test/<timestamp>/. Online auto-calibration still searches test/zero, test/left and test/right; redirect these to the matching folders in ../data/.